CHECK POINTS BEFORE STARTING PRODUCTION

- Make sure that outer box-making machine (BAR) is ready before starting the automat machine. Box glue pot heater to on 50 to 60 minutes before the starting of the machine.

- Paraffin reservoir coil should be opened earlier in suitable time so that paraffin remains in melted condition before starting of the machine. After melting pump to run.

- Paraffin in carriage tank should be in melted condition. After melting pump to run.

- Make sure that all parts such as head composition tank, applicator and all others are perfectly fitted. Then put head composition.

- Splint filling in FOD should be in right condition. If any abnormality is there please set right the same immediate.

- Check machine reserve bin with right size of splints.

- Make sure that outer box and inner box are with right shape. Check that those are falling freely through bows.

- Make sure that hot water level of head composition is perfect level. Then check electrical resistance pump are working perfectly. Head composition should be ready in time.

- Make sure wrapping machine VAT, inner box machine JUL is read. Glue/PVA should be kept in ready condition.

- Compressor should run in time.

- Air-water mist should run in dipping machine.

- Make sure all steam lines are working perfectly.