MAINTENANCES AND REPAIRS

BETWEEN 15 JUNE-15 JULY 2002

CONTENT

| 1. | Boiler Room | 3 |
| --- | --- | --- |
| 2. | Splint Line | 3 |
| 3. | Ayşe Outer Box machine | 5 |
| 4. | Ayşe Dipping Unit | 5 |
| 5. | Ayşe Match Line | 5 |
| 6. | Fatma Outer Box machine | 6 |
| 7. | Fatma Dipping Unit | 6 |
| 8. | Fatma Match Line | 5 |
| 9. | Cemile Outer Box machine | 7 |
| 10. | Cemile Dipping Unit | 7 |
| 11. | Cemile Match Line | 7 |
| 12. | Cemile Wrapping Machine | 8 |
| 13. | Gülin Outer Box machine | 9 |
| 14. | Gülin Dipping Unit | 9 |
| 15. | Gülin Match Line | 9 |
| 16. | Gülin Wrapping Machine | 9 |
| 17. | Sibel Outer Box machine | 10 |
| 18. | Sibel Dipping Unit | 10 |
| 19. | Sibel Match Line | 10 |
| 20. | Sibel Wrapping Machine | 12 |
| 21. | Zeynep Outer Box machine | 12 |
| 22. | Zeynep Dipping Unit | 12 |
| 23. | Zeynep Match Line | 12 |
| 24. | Zeynep Wrapping Machine | 13 |
| 25. | Head Comp. & Striking Surface Comp. | 14 |
| 26. | Varimatch | 14 |
| 27. | Chambon 500 | 15 |
| 28. | Inner Box Machines | 16 |
| 29. | Electric | 19 |
| 30 | Other | 20 |

1. Boiler Room

| 1 | Boilers and pressure vessels were prepared for periodic pressure test (Tests have been implemented by third party, MMO- Mechanical Engineering Association). Safety valves were adjusted to their set points. |
| --- | --- |
| 2 | Expansion tank of heating installation was cleaned. |
| 3 | Inside of dust cyclone was cleaned |
| 4 | Chimney filters were cleaned. |
| 5 | Water softening device was cleaned. |
| 6 | At number 4 Boiler, some bottom grills were changed. |
| 7 | Drain pumps at pumping room and boiler room were cleaned. |
| 8 | Excessive elbows were canceled at heating pipe installation. |
| 9 | At number 4 Boiler, front cover was changed. |
| 10 | At number 4 Boiler, discharging valve was changed. |
| 11 | At number 3 Boiler, manhole cover was changed. |
| 12 | At number 4 Boiler, spring type safety valve was renewed. |

2. Splint Line

|  | LINE 1 |
| --- | --- |
| 1 | Log conveyor bearings were changed. One broken bearing support was welded. |
| 2 | Missing grease fittings were completed. |
| 3 | Reductor coupling of log carriage was changed, keyways were renewed. |
| 4 | Chain and chain gears (½ 30 teeth) of log carrriage were changed. |
| 5 | Motor guide of log sawing machine was renewed. |
| 6 | Safety cover of sawing machine was changed. |
| 7 | Bushings of log stripping machine were renewed. Gearbox was cleaned and filled up with oil no 90.Oil level indicator was changed. |
| 8 | Log revolving gears ½ 3” 11 teeth (2) of stripping machine was changed. |
| 9 | Log revolving belt of stripping machine was changed 17x800. |
| 10 | Knives fixture of stripping machine were welded and then milled (4). |
| 11 | Wind flaps of stripping machine were welded and milled (8). |
| 12 | Fan filter of stripping machine was cleaned with pressure air. |
| 13 | Log revolving bushing rollers of stripping were renewed (3). |
| 14 | Main motor belts of stripping machine were renewed 17x2650 – 4. |
| 15 | Bearings were changed at log lifting shaft gears of peeling machine 6206 – 2 .New shaft was mounted. |
| 16 | Carriage belts of peeling machine were renewed 10x1300 – 2. |
| 17 | Bearings of carriage were changed 6008- 2. |
| 18 | Pres arm bearing of veneer piling unit was changed C-E –18-1. |
| 19 | Bearings of bottom sliding guide at splint chopping machine were changed NK-30/20 4. |
| 20 | Two guide shafts were mounted (30– with grease fittings). |
| 21 | Upper pres guide and screws of chopping machine were renewed. |
| 22 | Eccentric lever bushings and shaft of chopping machine were changed. |
| 23 | Eccentric lever bearings of chopping machine were changed Fag-10-36-34 A, 2. |
| 24 | The following group gears were changed; 26 teeth, 1 48 teeth, 1 151 teeth, 1 52 teeth, 1 48 teeth, 1 |
| 25 | Gears under peeling unit were changed CBK-38, 1 CBK-37, 1. |
| 26 | Log clamp unit was renewed completely: CBK-726-2 CBK-1771 CBK-1440-3 CBK-1963 CBK-25 CBK-1766 CBK-2073 TR-95x120x12 CBH-26 CBK-1960 CBH-32 CBH-28 CBK-23 CBK-22-3 CBK-737-2 CBK-24 CBK-738 SKF- 23024 + H – 3024 – 1 SKF= 51124 – three team SKF= 51214 – two team SKF NU 219 – 1 |
| 27 | Fıv sieving of Sibel and Emine were dismantled, cleaned and mounted. Sibel sieving plate lever was changed. |
| 28 | Line 1 dryer steel band was lied down and cleaned and repaired. |
| 29 | Line 1 dryer transfer ducts were repaired. |
| 30 | Line 1 dryer side sticks were renewed, missing nuts were completed. |
| 31 | Line 1 dryer bottom steel plates were cleaned. |
|  | LINE 2 |
| 32 | Front bearings of stripping machine were changed 1314. Ø35 shafts were changed |
| 33 | 2.Log revolving chain (1”), gears (1” 11teeth, 4), pulleys (3) and V belts (90,100,110) were changed 17x800. |
| 34 | Sawing machine bearings were changed 1311 K- 2. |
| 35 | Cover sealing and screws of debark transfer duct were changed. |
| 36 | Knives fixtures of stripping machine were changed. |
| 37 | Brass gears under veneer pulling unit were changed. |
| 38 | Chopping machine eccentric shaft and bushing were changed. |
| 39 | Gearbox driving chopping center have been dismantled, cleaned and bearings were controlled. |
| 40 | Knives fixture beam and pp were changed. |
| 41 | Two gears on veneer piling unit were changed. (3/4” 25 teeth) |
| 42 | Bearings on veneer transfer beam were changed. (2207) |
| 43 | Log carriage rollers were changed (4).Band bearings of log carriage were changed.(4 1210 K). |
| 44 | Veneer transfer chains and gears were changed. 5/8 - 5/8 25 teeth. 2 5/8 18 teeth 2. |
| 45 | Front side veneer transfer chains and gears were changed. ¾” 20 m. ¾” 23 teeth, 2 and 24 teeth, 2. |
| 46 | Chain rails of veneer transfer were changed. 10x10 and 10x30, each 8 meter. |
| 47 | Upper steel band of no 2 dryer was repaired. Missing cover rubber sealing were completed. |
| 48 | Fatma Fiv machine was dismantled, cleaned, bushings have been changed. Reductor bearings were changed. (6201) |
| 49 | Cemile Fiv machine was dismantled, cleaned, bushings were change. Broken sieves were repaired. Worn shaft was welded and turned .Motor pulley and belts were changed. (13x1100 ve 13x1300) |
| 50 | Gülin and Zeynep Fiv machine was dismantled, cleaned. Broken sieves were repaired. |
| 51 | No 1 dryer, steel band was repaired, some of them were changed. |
| 52 | No 1 veneer cutting shaft was welded and turned ,new bushing was mounted. |
| 53 | Variator pulley was mounted at chopping machine motor. 230 d/d |
| 55 | Line 2 speed were arranged to 230 d/d |

3. Ayşe Outer Box Machine

| 1 | Main shaft bearings were controlled. |
| --- | --- |
| 2 | Lower pulling shaft chains were changed (2). |
| 3 | Upper pulling shaft and 4 bearings were changed (6000). Delrin gear was changed. |
| 4 | Press bearings were changed 6, 625). |
| 5 | Side parts were repaired. |
| 6 | Glue pot bushings were controlled. |
| 7 | Reservoir sides were welded and then grinded. |
| 9 | Skillet transfer belt was changed. |
| 10 | Feeding finger and connection shaft were changed. Bearings were controlled. |
| 11 | Right finger was repaired. |
| 12 | Magnetic coupling was cleaned. Worn keyway was renewed. |
| 13 | Upper pres beam was changed. |
| 14 | Skillet discharge adjustment part was grinded. |
| 15 | Magazine cylinders were repaired. |
| 16 | Pulley keyway of magazine arm driving shaft was renewed. |
| 17 | New for forming bar is used. |

4. Ayşe Dipping Unit

| 1 | Sliding rail rollers were changed.. |
| --- | --- |
| 2 | Compound press arm bearings were changed (6008 2RSR, 4). |
| 3 | Compound press cam roller was changed (Nast 25 , 2). |
| 4 | Main shaft bearing was changed (6208 2RSR, 1). |
| 5 | Dipping band roller bearings were changed (UC 207, 4). |
| 6 | Gearbox shaft keyway was changed. |
| 7 | Hot water coircilation hoses were changed. |
| 8 | Dipping table bottom beams were renewed. Beam screws were changed. |
| 9 | Keyway of table driving chain gearwas changed. |
| 10 | Table pres profile spring was changed. |
| 11 | Compound shaft keyway was changed. |
| 12 | New bushing was mounted to main shaft driving gear. Shaft keyway was changed. |

5. Ayşe Match Line

| 1 | Outer box plunger brass were changed (4, 20x28x30) |
| --- | --- |
| 2 | Outer side outer box band steel was changed. (1) |
| 3 | ROG 4 main bearings were changed ( KK 1904, 5) |
| 4 | ROG 4 main bearings were changed (JTM 9289 , 4) |
| 5 | Main shaft chain gear was changed. |
| 6 | Filling unit shaft was welded, two bearings were changed. (3210 2RSR, 1213 K) |
| 7 | Outer side spring connection shaft bushings were changed. (4, 18x20x20) |
| 8 | Filling unit arm shafts were renewed. 16x30 ,4.Cubic bushings were changed (4). |
| 9 | Filling unit vibration bearings were changed. (1209 K, 4) |
| 10 | Control table chain was changed (5/8x3/8, 6 metre and 1/2x3/8, 2 metre). |
| 11 | Ejection unit gear bearings were changed. (2, 6012 2RSR) |
| 12 | Bar mat pulling gear bearings under filling unit were changed. (2, 6209 2RSR) |
| 13 | Filling unit arm plunger support was grinded. (1) |
| 14 | Control table box pulling part bushings.( 20x25x40, 2 - 20x25x25 , 1) and shafts20x580, 20x500 were changed. |
| 15 | Control table gear bearings were changed. (6203, 2) |
| 16 | Gear for ejection and bar mat pulling shaft was divided into two parts. Core of coupling was renewed. |
| 17 | Filling unit bearings werte changed.(Nast 17, 4) |
| 18 | Inner box down feed bows,box stopping air cylinders were changed with the smaller diameters. (Piston 25x25, 2) |
| 19 | Filling unit splint cleaning brushes were changed.(8) |
|  | KEB5A + KED4 + FOD |
| 20 | Shaft bearings were changed.(UC 206, 8) |
| 21 | Vibration bearings were changed.(2204, 2) |
| 22 | Vibration arm bearings were changed (1204 ,1) |
| 23 | Pinion gear bearings were changed.(6006, 2) |
| 24 | Brush bearings were changed.(1201, 1) |
| 25 | Bearings were changed (6201, 1 – 6202, 2 - 6004, 2) |
| 26 | 6005 rulman 2 adet Ked4 gerdirme rulmanları değişti. |
| 27 | Ked4 band motor belt wwere changed.(13x1300 V belt, 2) |
| 28 | Fod vibration unit bearings were changed (2209 K , 1) . |
| 29 | Fod vibration unit shaft bearings were changed (2208 K , 1). |
| 30 | Fod vibration unit arm bearings were changed (1212 TV, 2) |
| 31 | Fod vibration unit brush bearings were changed (2205, 2) |
| 32 | Keb 5A motor rear gear was changed (½” 21 teeth ,1) |
| 34 | Brush and channel driving shaft gear was changed (½” 57 teeth, 1) |
| 35 | Fod side bushings were changed.(4) |
| 36 | Chains on Fod and Keb 5A units were changed (½”, 3/4”, 5/8”) |
| 37 | Ked 4 wide band bearings were changed (UC 205, 4) |
| 38 | Ked 4 distrubition pin was renewed. |
| 39 | Broken channel and band parts were repaired. |

7. Fatma Outer Box Machine

| 1 | Main shaft bearings were controlled. |
| --- | --- |
| 2 | Upper pulling shaft chains were changed (2). |
| 3 | Upper pulling shaft and delrin gear was changed. |
| 4 | Glue pot bushings were controlled. |
| 5 | Chain bottom guides were changed. |
| 6 | Feeding finger and connection shaft were changed. Bearings were controlled. |
| 7 | Magnetic coupling was cleaned. Worn keyway was renewed. |
| 8 | Upper pres beam was changed. |
| 9 | Press disc connection bushings were changed. |
| 10 | Chain pinion gear was changed. |
| 11 | Skillet discharge adjustment part was grinded. |
| 12 | Delrin gear was changed (1). |
| 13 | Motor pulley was changed (17x1425 1). |
| 14 | Chain cleaning dust brush was changed. |
| 15 | Main drive chain gear was repaired. |
| 16 | Upper feeding shaft chain pinion was changed. |
| 17 | Magazin cylinders were repaired. |
| 18 | New for forming bar is used. |

8. Fatma Dipping Unit

| 1 | Bar mat gear was changed. |
| --- | --- |
| 2 | Hot water circulation pipe was cleaned. |
| 3 | Hot water circulation hoses were changed. |
| 4 | Sliding rail rollers were changed (6204, 4) |
| 5 | Dipping band roller bearings were changed (UC 207, 4). |
| 6 | Dipping table bottom beams were renewed (160x60x8). Beam screws were changed. |
| 7 | Beam screws were changed. (100x10, 8). |

11. Cemile Outer Box Machine

| 1 | Main shaft bearings were controlled. |
| --- | --- |
| 2 | Lower pulling shaft chains were changed (2). |
| 3 | Upper pulling shaft and bearing were changed (6000). |
| 4 | Glue pot bushings were controlled. |
| 5 | Chain bottom guides were changed. |
| 6 | Broken pins under guides were changed. |
| 7 | Mechanic coupling was repaired. |
| 8 | Feeding finger eccentric shaft and bearing were changed (NKI 16/9). |
| 9 | Broken bearings at pinion gear were changed (6204 – 2). |
| 10 | Chain gear was repaired. |
| 11 | Mainframe screws were renewed. |
| 12 | Magazine cylinders were repaired. |
| 13 | Mechanic coupling brake coils were changed. |
| 14 | Skillet discharge unit was welded and repaired. |
| 15 | Chain dust brush was changed. |

12. Cemile Dipping Unit

| 1 | Compound press arm shaft bushings were changed (40x50x55, 4). |
| --- | --- |
| 2 | Compound press arm bearings were changed (6008 2RSR, 4). |
| 3 | Main shaft bearing was changed (6208 2RSR, 2). |
| 4 | Gearbox shaft keyway was changed. |
| 5 | Compound press bushings was changed.(4) |
| 6 | Dipping band roller bearings were changed (UC 207, 4). |
| 7 | Sliding rail rollers were changed (6204, 4) |
| 8 | Hot water circulation pipe was cleaned. |
| 9 | Hot water circulation hose was cleaned. |
| 10 | Keyway of table driving chain gearwas changed. |
| 11 | Beam screws were changed. |
| 12 | Dipping cam-driving gears were changed (KL 11587-KL 11614). |
| 13 | Table pres profile spring was changed. |

13. Cemile Match Line

| 1 | Outer side, Rog 4 band wheel has been changed. |
| --- | --- |
| 2 | Outr side spring connection shaft bushings were changed. (4, 18x20x20) |
| 3 | Filling unit arm plunger parts were controlled. |
| 4 | Control table mechanism bushingwas changed.(30x34x30) |
| 5 | Bushing (20x23x25, 6) of box pulling to the contro table and shafts (20x500) were changed. |
| 6 | Filling unit vibrator bearings were changed. (1209 K, 4 - 2203 2RS, 4) |
| 7 | New aluminium down feed bows was installed. (18) |
| 8 | Inner box guides coming to ROG 4 were changed. (2, RO 139 73) |
| 9 | Nast 25 bearing were changed with 3 shafts. |
| 10 | ROG 4 bands were changed (4) |
| 11 | Filling unit cams were repaired (KK 7340 , 2 - 47) |
| 12 | Filling unit cam bearings were changed (NUTR 1747, 4) |
| 13 | Five cams were repaired (52). KK 9440-9439-9443-11772-15191 |
| 14 | Outer box plunger cam rollers were changed. (Nast 25 50 2) |
| 15 | Inner box stopping air cylinders at down feed bows were changed with the smaller diameters.(25x25, 2). |
| 16 | Filling unit splint cleaning brushes were changed (8). |
| 17 | Shaft chain driving ROG4 was changed (3/4 double order 35cm). |
|  | KEB5A + KED4 + FOD |
| 18 | Keb 5A shaft bearings were changed (UC 206, 10) |
| 19 | Brush bearings were changed (1204, 1 and 1201, 1). |
| 20 | Ked4 band bearings were changed (UC 205, 4). |
| 21 | Keb5A vibration bearings were changed (2204, 2). |
| 22 | Shaft gear (½” 57) driving brush was changed. |
| 23 | Bearings of vibration arm driving channels were changed (1207, 1). |
| 24 | Keb 5A motor belt was changed (13x1175 V belt,1). |
| 25 | Ked4 band motor belt was changed.(13x1250 V belt, 2). |
| 26 | Ked4 band tensioning roller was changed (6004, 2 and 6002, 2). |
| 27 | Fod vibration arm bearings were changed (1212, 2). |
| 28 | Fod vibration shaft bearings were changed (2208,1). |
| 29 | 1209 rulman 1 adet Fod sarsak milinin yatak rulmanı değişti. |
| 30 | Splint leveling unit,cover shaft bushings were changed.(35, 40 ,1 for each). |
| 31 | Fod and Keb 5A chains were changed (½”, 5/8”, ¾”). |
| 32 | Fod side shaft bushings were changed (4, 2205). |
| 33 | Broken channels and other parts were repaired. |
| 34 | New cleaning device after 1.st box closing unit. |

14. Cemile Wrapping Machine

| 1 | Box holdig cam roller(KR 40 PP) and bushings(30x30, 2) on the shaft were changed. |
| --- | --- |
| 2 | Box making cam roller (KR 40 22) and bushings (30x30, 2) on the shaft were changed. |
| 3 | Shaft,roller and fittinf inside box making arm were changed. |
| 4 | Lower aluminium rollers, shafts, bearings (608-2RS, 4) and upper small gears of PP pulling belts were changed. |
| 5 | Side folding arm bearins (KR 40 PP) were changed. |
| 6 | Lover folding arm bearing (KR 40 PP) was changed. |
| 7 | Upper folding arm bearing (KR 40 PP) was changed. |
| 8 | Pipe shaft and all bushings of side foldings were changed. (12x14x15 ve VA 14619). |
| 9 | Upper sleeve shaft was changed (35x20) 250 mm. |
| 10 | Lower belt moving above box plunger arm was changed. (VA 12440 – 1 adet). |
| 11 | Box plunger cam roller(KR 35 PP) was changed. |
| 12 | Upper press sleeve inner shaft was changed (VA 12562). |
| 13 | PP unit pulling belt bearings were changed (4 ,608). |
| 14 | Box stopping fingers were changed (VA – 12909 – 1) (VA – 12908 – 1). |
| 15 | Box stopping finger guides were changed. (VA – 12904 – 2 Adet) |
| 16 | Box carriage belts were changed (65x2075 – 2 adet). |
| 17 | PP unit long pulling belts was changed (990 mm, 2). |
| 18 | PP unit short pulling belts were changed (560 mm, 2). |
| 19 | Upper sleeve shaft was changed (VA – 12631 – 1 adet). |
| 20 | General adjustmens were done. |

15. Gülin Outer Box Machine

| 1 | Main shaft bearings driving coupling were changed (2204 – 2). |
| --- | --- |
| 2 | Eccentric moving pp gear bearing was changed (NKI 16/9). |
| 3 | Transfer chains of lower pulling shafts were changed (2). |
| 4 | Skillet discharge unit was welded and repaired. |
| 5 | Skillet discharge transfer part was grinded. |
| 6 | Lower pulling shaft and bearings were changed (6000 – 1). |
| 7 | Chain bottom guides were changed. |
| 8 | Spring pushing bar was changed (Hu 1503 – 10). |
| 9 | Magazine cylinders were repaired. |
| 10 | Pressing sprigs were changed (4). |
| 11 | Main drive pulley key was changed. Worn shaft was welded and turned. |
| 12 | Chain dust brush was changed (2). |
| 13 | Chain tensioning gear bearing was changed (6203, 1). |

16. Gülin Dipping Unit

| 1 | Sliding rail rollers were changed (6204, 4). |
| --- | --- |
| 2 | Dipping band roller bearings were changed (UC 207, 4). |
| 3 | Hot water circulation pipe was cleaned with acid bath. |
| 4 | Hot water circulation hose was cleaned. |
| 5 | Beam screws were changed (100x10). |
| 6 | Main drive gear chain was changed (1/2). |
| 7 | Bar mat driving cam was changed (RO – 18782 A). |
| 8 | Dipping band cam bearings were changed (Nast 25, 12 adet. |
| 9 | Bar mat cleaning brushes weer changed. |

17. Gülin Match Line

| 1 | Inner side outer box feeding unit,guide was changed.(RO 25778). |
| --- | --- |
| 2 | Control table: bevel gear keyway has been renewed. |
| 3 | Rog 4 guide plates have been changed. |

18. Gülin Wrapping Machine

| 1 | Machine was cleaned with pressure air and diesel. |
| --- | --- |
| 2 | New PP unit was installed. |
| 3 | Outomatic arm was dismounted firstly, and then mounted again by controlling all parts to provide prorer working. |
| 4 | Bushings and shafts of box pulling conveyor roller and gears were changed (4 shafts,2 bushings). |
| 5 | Box stopping finger bushings were changed. (2) |
| 6 | Side folding bushings were changed. |
| 7 | Upper folding connection unit was revised and mounted. |
| 8 | Shaft and roller of automatic arm were changed. |
| 9 | Upper and lower folding cams were renewed. |
| 10 | Chain gear driving new PP unit were changed. |
| 11 | PP unit heaters were changed. |
| 12 | Roll placing apparatus bushings were changed. |
| 13 | General adjustmen was done. |

19. Sibel Outer Box Machine

| 1 | Magazine air sealings were changed. |
| --- | --- |
| 2 | Manyetik coupling shaft keyway was changed. |
| 3 | Lower pulling chains were changed. |
| 4 | Shaft bearing driving finegr was changed (INA 1216). |
| 5 | Main shaft bearings and cams were controlled. |
| 6 | Upper pulling shaft and 4 bearings were changed (6000, 2). Delrin gear was changed. |
| 7 | Eccentric delrin gear that is driving feeding finger was changed. |
| 8 | Skillet feeding sides were welded. |
| 9 | Skillet adjustment apparatus was grinded. |
| 10 | Transfer chain was cleaned. |
| 11 | Chain transfer rails and pushing bars were changed (12). |
| 12 | Chain rails were changed. |
| 13 | Main driving pulley keyway was repaired.New bushings were mounted. |
| 14 | Coupling belt was changed. |
| 15 | Dust brushes were changed. |
| 16 | Disk pres part was grinded. |

20. Sibel Dipping Unit

| 1 | Compound press arm bearings were changed (6008, 2). |
| --- | --- |
| 2 | Compound press arm bearings were changed (4, 44x20). |
| 3 | Compound press arm driving shaft bushings were changed (2 adet 50x37). |
| 4 | Dipping band roller bearings were changed (UC 207, 3). |
| 5 | Compound press arm spring collars were changed. |
| 6 | Compound press cam shaft bushings were changed (4, 50x55x40). |
| 7 | Beam screws were changed (8). |
| 8 | Hot water circulation pipe was cleaned with acid bath. |
| 9 | Hot water circulation hose was cleaned. |
| 10 | Shaft gearbox bearings were changed (2, 6010). |
| 11 | Main shaft bearings were changed (4,Nast 25). |
| 12 | Compound gearbox shaft bearings were changed (6214-6213, 2). |
| 13 | Shaft keyway was repaired. |
| 14 | Bar mat driving shaft bearings were changed (1, 6208). |
| 15 | Sliding rail rollers were changed (4, 6204). |
| 16 | Main shaft chains were changed (1/2 single order 4.5 m, double order ½ 1 m). |
| 17 | Dipping table bottom beams were renewed (4). |
| 18 | New bar mat tensionings were installed (2). |

21. Sibel Match Line

| 1 | Lower spring guide was changed (RO 25730). |
| --- | --- |
| 2 | Bearings on the shaft driving gearbox under ROG 4 were changed (1213 K and 3210 B). |
| 3 | Gearbox shaft gear was changed (216). |
| 4 | Pinion gear that is driving main shaft was changed (KK 9152). |
| 5 | Pinion gear bushing (30x34x30) was changed.Shaft (K.K 11543) was changed. |
| 6 | Main shaft bearings were changed (UC 212 , 5 and 1213 K, 3). |
| 7 | Coupling was changed (K.12 2121).Keyway was renewed, |
| 8 | Coupling disengaging arm bearing was changed (Nast Y 10 PP, 1). |
| 9 | Index side ROG4 band rails were changed.(RO 25623 inner, RO 25624 outer). |
| 10 | Filling unit spring side,ROG4 band rails were changed (RO 25657 outer, RO 25658 inner). |
| 11 | Filling unit inlet band rails were changed. |
| 12 | Filling unit band rails were changed (inner-outer) |
| 13 | Spring driving cams (JTM 9443) and bearings (Nast 25, 2) were changed. |
| 14 | Filling vibrating bearings were changed (2209 K, 4) |
| 15 | Filling vibrating eccentric teflon guides were changed (4). |
| 16 | Filling vibrating eccentric bearings (2203 2RS, 1) |
| 17 | Filling unit plunger driving cam (KK 6581) and bearing (INA 1542) were changed. Brass cubes were changed (2). |
| 18 | Filling unit outr cams were repaired (42 Nast 17, 2). |
| 19 | Bearings were changed (TNA 1542, 2). |
| 20 | Filling unit arm bearings (NK 2220, 3) were changed. Brass bushings were changed (4). |
| 21 | Filling unit plunger bass cubes were changed to new system. |
| 22 | Bar mat pulling brass arm bushing was changed.Bar mat pulling arm was changed. |
| 23 | Control table, box pulling finger shafts (25) and bushings (25x28x30) were changed. |
| 24 | Control table driving main chain was changed. (3/8’ 5/8) |
| 25 | Control table, box closing shaft, chain gear and bearings (6206, 2) |
| 26 | Cam bearings pulling boxes to control table were changed (Nast 25 2). |
| 27 | Box closing ifnger cam bearings were changed (Nast 25, 2). |
| 28 | Conrol table, bevel gear keyway was renewed. |
| 29 | Outer side outer box feeding guides were changed (RO 25776). |
| 30 | Covers for outer box feeding unit was installed. |
| 31 | İnner box press joints were changed. |
| 32 | Main shaft chains were changed (½”, 3-1/4 double order). |
| 33 | Broken or missing pipes of automatic lubrication system were repaired. |
| 34 | Band wheels from index side were changed. |
|  | KEB5A + KED4 + FOD |
| 35 | Keb5A shaft bearings were changed (UC 206, 10). |
| 36 | Pinion gear bearings were changed (6006, 2). |
| 37 | Vibration arm bearings were changed (1207, 1). |
| 38 | Vibration arm bearings were changed (2204, 2 ) |
| 39 | Keb5A brush bearing was changed (1201,1 – 1204,1). |
| 40 | Keb5A motor gear was changed (½” 21 teeth, 1). |
| 41 | Keb5A motor belt was changed (13x1225 V belt, 1). |
| 42 | Ked4 band motor belt was changed (13x1100 V belt, 1). |
| 43 | Ked4 band bearings were changed (UC 204, 2). |
| 44 | Ked4 band bearings were changed (UC 205, 2). |
| 45 | V belt was changed (17x1450 V belt, 1). |
| 46 | Ked4 tensioning roller bearings were changed (6204,2 and 6302, 2). |
| 47 | Keb5A chains were changed. Splint leveling brus system was installed to Fod unit. |
| 48 | Two bearings were used for splint leveling brush system (1304, 2). |
| 49 | Broken plates were repaired at Fod unit. |
| 50 | Band wheels have been changed. |

22. Sibel Wrapping Machine

| 1 | Wrapping machine that came from Spain was replaced with the one that was at Sibel Line.The machine was in good condition,so no needed any maintenance. |
| --- | --- |

23. Zeynep Outer Box Machine

| 1 | Skillet feeding sides were grinded. |
| --- | --- |
| 2 | Skillet adjustment apparatus was grinded. |
| 3 | Main shaft bearings driving eccentric were changed. (2, 2204) |
| 4 | Coupling bearing was changed. (6004) |
| 5 | Eccentric bearing driving feeding finger was changed. INA-1216 |
| 6 | Upper pulling shaft and 4 bearings were changed (6000). Delrin gear was changed. |
| 7 | Upper pulling shaft pp gear was changed. |
| 8 | Lower pulling shaft chains were changed (2). |
| 9 | Transfer chain was cleaned. |
| 10 | Eccentric pp gear was changed. |
| 11 | Transfer rails were changed (27). |
| 12 | Glue pot bushings were changed. |
| 13 | Spring pushing bars were changed (27). |
| 14 | Main motor shaft was welded and turned. Keyway was changed. |
| 15 | Big pulley was repaired. |
| 16 | Rail pushing screws were changed (10). |
| 17 | Dust brush was changed (2). |

24. Zeynep Dipping Unit

| 1 | Dipping band roller bearings were changed (UC 207, 4). |
| --- | --- |
| 2 | Dipping table guide bearings ewre changed (6204,4). |
| 3 | Compound press arm bearings were changed (6008, 4). |
| 4 | Compound press arm shaft bushings were changed (50x40, 2). |
| 5 | Dipping table bearing block was changed (1). |
| 6 | Spring press bushings on compound arm were changed (50x37 , 2). |
| 7 | Main shaft bearings were changed (6208, 2). |
| 8 | Shaft bearings were changed (6007, 2). |
| 9 | Guide spring of compound press arm was changed. |
| 10 | Dipping table was replaced with the new one. |
| 11 | Dipping table bottom beams were renewed. |
| 12 | Hot water circulation hose was cleaned. |
| 13 | Compound resevoir was renewed. |
| 14 | Compound pumping rubber was changed. |
| 15 | Connection rot of hot water circulation pipe was repaired. |
| 16 | Shaft gear keyway was repaired. |
| 17 | Dipping table chain gear key was renewed. |
| 18 | Hot water circulation pipe was cleaned with acid bath. |

25. Zeynep Match Line

| 1 | Bearings on the shaft driving ROG 4 were changed (1213 K, 1 and 3210 B, 1). |
| --- | --- |
| 2 | Chain gear on the shaft driving ROG 4 was changed (208). |
| 3 | Chains between main motor and ROG 4 were changed (½”, 3-1/4” double order 5 part). |
| 4 | Brass bushings on box pulling arms and their shafts (16x25 and 25). |
| 5 | Box pulling arm cam brpllers were changed (NAST 25, 2) |
| 6 | Box closing finger bushings were changed to standard bushing system (25x28x30, 8) |
| 7 | Brass bushings of upper sprinh frame were changed (4). |
| 8 | Cam bearings of box transfer finger to contol table was changed (NAST 25, 1) |
| 9 | Cam (RO 9979) and cam roller (Nast 17) driving filling arm plunger were changed. Two brass cubes were changed (15x25, 2). |
| 10 | Filling unit guide plunger bushings were changed (30x34x20, 8). Dust sealings were changed (30x40-5/8, 8). |
| 11 | Brass cubes on filling unit arm were changed to new system (10x25, 4). |
| 12 | Filling unit arm bearings were changed. (NK 22-20) |
| 13 | Filling unit vibration bearings were changed.(1209 K, 4) |
| 14 | Outer side vibration eccentric bearing was changed (2203 2RS). Teflon bushing was changed. |
| 15 | Filling unit vibrator motor belt was changed. (13x925) |
| 16 | ROG 4 group gear coupling 82121) and cross gears were changed (KK 47449) and (KK 6282). |
| 17 | Gear (216) and pinion gear (K.K 9152) on ROG 4 driving shaft were changed. |
| 18 | Outer side outer box feeding shaft (20) and cam roller (Nast 25) were changed. |
| 19 | İinner and outer box feeding shaft (20) and brass bushings were changed (4). |
| 20 | Coupling chain gear on the shafr driving control table was changed RO 17227). |
| 21 | Control table bevel gears were changed (RO 17619, RO 17618). |
| 22 | Control table box closing cam chain was changed (½”). |
| 23 | Control table, bearings at box pulling finger were renewed. |
| 24 | Box pulling chain from control table to KU1 band was changed. |
| 25 | Air cylinders driving control table plungers were changed. (25, Stroke 100, 2) |
| 26 | Rails under spring holding arm were changed. (İnner RO 17011,outer RO 17012) |
| 27 | Rails under filling unit vibration inlet were changed. (Inner RO 25656, outer RO 25655) |
| 28 | İinner box stopping air cylinders were changed with the smaller diameters. Bushings and joints were changed. |
| 29 | Filling unit cams were controlled. |
|  | KEB5A + KED4 + FOD |
| 30 | Pinion gear bearings were changed (6006, 2) |
| 31 | Keb5A shaft bearings were changed (UC 206, 6). |
| 32 | Ked4 band tensioning roller bearings were changed (6302, 4). |
| 33 | Keb5A chain pinion gear bearings were changed (6201, 2). |
| 34 | Vibration arm bearings were changed (2204, 2). |
| 35 | Channel vibration arm bearing was changed (1207, 1). |
| 36 | Chain gears were changed (½” 57 teeth and ½” 21 teeth). |
| 37 | Studs were changed (M14 x100, 3). |
| 38 | Ked4 band bearings were changed (UC 204, 2). |
| 39 | Ked4 band bearings were changed (UC 205, 2). |
| 40 | Keb 5A motor belt was changed (13x1050 V belt, 1). |
| 41 | Ked 4 band motor belt was changed (17x1425 V belt, 1). |
| 42 | Box holding systems were installed to Sibel, Gülin and Fatma lines. |
| 43 | New aluminuim down feed bows were installed to Ayşe and Fatma lines. |

26. Zeynep Wrapping Machine

| 1 | Machine was cleaned with pressure air and diesel. |
| --- | --- |
| 2 | New PP unit was installed. |
| 3 | Outomatic arm was dismounted firstly, and then mounted again by controlling all parts to provide prorer working. |
| 4 | Bushings and shafts of box pulling conveyor roller and gears were changed (4 shafts, 2 bushings). |
| 5 | Box stopping finger bushings were changed (2). |
| 6 | Side folding bushings were changed. |
| 7 | Upper folding connection unit was revised and mounted. |
| 8 | Upper and side folding cams were renewed. |
| 9 | Chain gear that is driving new PP unit were changed. |
| 10 | PP unit heaters were changed. |
| 11 | Roll fixture bushings were changed. |
| 12 | General adjustmen was done, covers were painted. |

28. Head Comp. & Striking Surface Comp.

|  | HEAD COMPOSITION PREPERATION |
| --- | --- |
| 1 | Head composition mill machine was repaired. Grinding stones were changed. |
| 2 | Diaphragm shafts, diaphragms and flanges of air pumps were maintained. Broken diaphragms were changed. |
| 3 | Oil and grease fittings of glue (PVA) mixture gearboxwere changed. |
| 4 | Air cylinder sealings no 1 and 2 skin glue tanks were changed (17x35x7, 2). |
| 5 | Pneumatic installation of no1 composition tank was repaired. |
| 6 | Ai,r cylinder sealings were changed (16x100x25, 1). |
| 7 | Parafin tank was renewed. The bigger one was installed with tre necessary armatures. |
| 8 | Gearbox oils of no 1,2,3 compound tank were changed. The motors were overlooked. |
| 9 | Raw material transfer conveyorwas controlled. |
| 10 | Isolation plate of premixli compound tank was renewed. |
| 11 | Water resorvoir tank was cleaned. |
| 12 | Necessary paintins were done at Head Cmpound unit including walls. |
| 13 | No 1,2 and 3 mixing tanks were maintened. Grease fittings were changed. |
|  | STRİKİNG SURFACE COMPOUND PREPERATION |
| 14 | Striking compound mixing bearings and sealings (16x35x8, 1) were changed. |
| 15 | Striking compound preparing tank was cleaned and maintenanced. |
| 16 | Impregnation tank mixture was maintenanced. |
|  | WATER TREATMENT UNIT |
| 17 | Waste chlorate water pump sealing (25x35x7) and coils were changed. |
| 18 | Waste chlorate water tank was cleaned. |
| 19 | Mud pump sealings and coils were changed (40x55x8). |
| 20 | Head composition and striking compound wells were cleaned. |
| 21 | Gearbox and motors at Water treatment unit were maintenanced and painted. |
| 22 | Biologic waste treatment pumps were maintenanced. |
| 23 | Water treatment unit walls were painted. |

29. Varimatch

| 1 | Swinging roller bearings were changed (4, UC 205 and 2, E.E.8). |
| --- | --- |
| 2 | Sheet feeding suction head was dismounted,bushings and pins were changed. |
| 3 | Sheet feeding belts were converted to roller bearings type (8, 607). |
| 4 | Slitting shaft bearings were changed (2, 6209 and 1, 6208). |
| 5 | Cutting knive bearings were changed (2, 6209 and 2, 6208). |
| 6 | Clamp shaft bearings were changed (6209, 2). |
| 7 | Driving shaft joints were controlled. Broken ones werechanged, balance adjustment were controlled. |
| 8 | Chain driving narrow band was changed. |
| 9 | Chain driving coupling was changed. |
| 10 | Chains driving cutting unit were changed (2). |
| 11 | Slitting cover bearings were changed (2, 6209). |
| 12 | Cutting unit cover bearings were changed (2, 6209). |
| 13 | Sheet transfer narrow band bearings were changed (6005, 2 and 6001, 1). |
| 14 | Sheet transfer narrow band joint and pin were changed. |
| 15 | Positioning bearings were changed (1203 K, 2). |
| 16 | Teflon band bearing sleeves wre renewed. Two rollers were coated with teflon. |
| 17 | Oil and bearings of gearbox driving no 1 transfer were changed (6007, 2). |
| 18 | Oil and bearings of gearbox no 2 were changed (6007, 2). |
| 19 | Oil and bearings of gearbox no 3 were changed (6007, 2). |
| 20 | Oil and bearings of gearbox no 4 were changed (6007, 2 and 6206, 2). |
| 21 | Oil and bearings of gearbox no 5 were changed (6007, 2). |
| 22 | Front cutting unit roller fixtures were changed (4). |
| 23 | No 1-roller shaft was changed. |
| 24 | No 2-roller shaft was changed. |
| 25 | No 3-roller shaft was changed. |
| 26 | No 4-roller shaft was changed. |
| 27 | Sheet pulling roller was grinded. |
| 28 | Sheet pulling roller shaft was changed. |
| 29 | Sheet pulling roller bearings were changed (2, 6206 and 1, 6305 and 6, 6005). |
| 30 | Cutting unit plates were straigthed. |
| 31 | Sheet feeding stop plate was welded. |
| 32 | Sheet feeding narrow bands were changed (4). |
| 33 | Dryer outlet roller bearings were changed (6005, 2). |
| 34 | Gears bringing skillets to grippers were changed (2). |
| 35 | Tension band shaft driving grippers was changed. |
| 36 | Striking surface unit coupling bearings were changed (6209, 1). |
| 37 | Tray drop arm shaft was changed. |
| 38 | Cutting unit bearings were changed (for 53 mm 6006, 4). |
| 39 | Scraping knife apparatus were remanufactured. |
| 40 | PP gear at pfosfor graving group was changed. |
| 41 | Rollers (4) and bushings (5x7x8, 8)on narrow band were changed. |
| 42 | Sheet carriage band was changed. |
| 43 | Phosphorus graving roller bearings were changesd (6008,1). |
| 44 | Swinging roller belt was changed (1100 H). Belt fixture frame and eccentric were renewed. |
| 45 | Phosphorus inlet platin bearings were changed (UC 206, 1). |

30. Chambon 500

| 1 | Phosphorus graving roller bearings were changed (2, 6209). |
| --- | --- |
| 2 | Brass worm gear driving phosfor graving roller-scraping knife was changed. |
| 3 | Bevel gears at no 2 gearbox were changed. Bearings were changed (2, 6209). |
| 4 | No 1 joints were controlled. |
| 5 | No 2 joints were controlled. |
| 6 | PP gears weer changed ar front pulling group (2). |
| 7 | Upper roller bearing places were renewed. |
| 8 | Front pulling roller was controlled. |
| 9 | No 2 gearbox was changed. |
| 10 | Gearboxes oil and gears were controlled. |
| 11 | Roller after 53 mm cutter was dismounted, controlled and mounted. |
| 12 | Roller transferring skillets to gripper was dismounted, controlled and mounted. |
| 13 | Phosphorus graving adjustment gear bearings were changed (608, 6). |
| 14 | Phosphorus graving worm gear bearing was changed (1, 6006). |
| 15 | Phosphorus graving worm gear bearing was changed (1, 6007). |
| 16 | Phosphorus graving worm gear shaft was changed. |
| 17 | No 1 cardboard pulling unit was controlled. |
| 18 | Roller at front cutting unit was dismounted, controlled and mounted. |
| 19 | Skellet pulling roller bearings at front cutting unit were changed (6003, 6 and 6004,1). |
| 20 | Pulling roller bearings were changed (6004, 2). Roller was dismounted, controlled and mounted. |
| 21 | PP ger bearings weer changed (6002, 4). |
| 22 | No 1-gearbox bearings were changed (6206, 2 and UKI 1586, 2).Oil sealings were changed (2). |
| 23 | No 1-gearbox bevel gears were changed. |
| 24 | No1, 2,3,4 paint printing units were controlled. |
| 25 | Water rollers were coated with chrome. |
| 26 | Lubrication oil tanks were controlled. |
| 27 | Cardboard automatic feeding machine gearbox was repaired. |
| 28 | Phosphorus compound tank side pp parts weer changed. |

31. Inner Box Machines

| Inner box machine no 1 |  |
| --- | --- |
| 1 | New punching tool are used. |
| 2 | Ju 1218 gear was changed (141, 1). |
| 3 | Ju 1368 gear was changed (130, 1) |
| 4 | Ju 1351 bottom track was changed (1). |
| 5 | Ju 1369 fiber gear was changed (Z 65-138, 1). |
| 6 | Ju 1219 gear was changed (140, 1). |
| 7 | Ju 1332 gear was changed (Z 21-131, 1). |
| 8 | Ju 1350 gear (129, 1) and upper track were changed. |
| 9 | Bearings were changed (6204, 2) |
| 10 | Upper creasing shaft bushings were changed (4). |
| 11 | Bearing was changed (6206 2RS, 2) |
| 12 | Inner box carriage was changed. |
| 13 | Fork was changed. |
| 14 | Shaft driving inner box carriage and bottom-upper track shaft was changed. |
| 15 | Delrin part supporting box carrriage was changed. |
| 16 | Bottom glue aplicator roller was changed. |

| Inner box machine no 2 |  |
| --- | --- |
| 1 | Ju 1368 gear was changed (130, 1). |
| 2 | Ju 1351 fiber bottom track was changed (1). |
| 3 | Ju 1585 fiber gear was changed(128, 1). |
| 4 | Ju 1332 gear was changed (1). |
| 5 | Bearings were changed (6009 2RS, 1). |
| S | Bearings were changed (6206 2RS, 2). |
| 7 | Upper creasing shaft bushings were changed (4). |
| 8 | Skillet carriage chains were changed (2). |
| 9 | Skillet chain wheel was changed (124, 2). |
| 10 | Ju 1218 gear was changed (141, 1). |
| 11 | Ju 1219 gear was changed (140, 1) |
| 12 | Ju 1369 fiber gear was changed (Z 65-138, 1) |
| 13 | Ju 1356 skillet folders were changed (2) |
| 14 | İnner box carriage was changed. |
| 15 | Fork was changed. |
| 16 | Delrin part supporting box carrriage was changed. |

| Inner box machine no 3 |  |
| --- | --- |
| 1 | Skillet carriage chains were changed (2). |
| 2 | Ju 1351 bottom fiber track was changed (1). |
| 3 | Upper track was changed. |
| 4 | Ju 1425 chain link was changed (51, 1). |
| 5 | Ju 1368 gear was changed (130, 1). |
| 6 | Ju 1369 fiber gear was changed (Z 65-130, 1). |
| 7 | Ju 1219 gear was changed (140, 1). |
| 8 | Ju 1218 gear was changed (141, 1). |
| 9 | Bearing was changed (6206 2R, 2). |
| 10 | Skillet carriage was changed. |
| 11 | Delrin part supporting box carrriage was changed. |
| 12 | Fork was changed. |
| 13 | Shaft driving skillet carriage and bottom track was changed. |
| 14 | Box forming plates were changed (2). |

| Inner box machine no 4 |  |
| --- | --- |
| 1 | Ju 121 gear was changed (141, 1). |
| 2 | Bearings were changed (6204, 4). |
| 3 | Ju 1219 gear was changed (140, 1). |
| 4 | Ju 1369 Fiber gear was changed (Z 65-138, 1). |
| 5 | Ju 1368 gear was changed (130, 1). |
| 6 | Ju 1351 fiber bottom track was changed (1). |
| 7 | Upper creasing bushings were changed. (4). |
| 8 | Box forming plates were changed (left-rigth). |
| 9 | Fork was changed. |
| 10 | Lower bottom box forming sides were changed. |
| 11 | Upper creasing press shaft was changed. |

| Inner box machine no 5 (new) |  |
| --- | --- |
| 1 | Pulley was installed to the main motor. |
| 2 | New bottom and top glue application rollers were installed. |
| 3 | Inner box machines were re-arranged. Compressed air pipe installation was done. |
| 4 | New band was installed. The band was made longer. |
| 5 | Ducts have been re-arranged. |
| 6 | Air leakages were controlled. |
| 7 | Fan was installed to the new machine. |
| 8 | New ducts were added to the system,and electric lightening installation was moved according to the new system. |
| 9 | Bundling fan motor belts weer changed. |

| Inner box machine no 6 |  |
| --- | --- |
| 1 | Bearing was changed (6206 2RS, 2). |
| 2 | Ju 1368 gear was changed (130, 1). |
| 3 | Ju 1369 fiber gear was changed (Z 65-130, 1) |
| 4 | Ju 1219 gear was changed (140, 1). |
| 5 | Ju 1218 gear was changed (141, 1). |
| 6 | The bottom track was changed (1). |
| 7 | Ju 1332 gear was changed (1). |
| 8 | Ju 1356 skillet folders were changed (2). |
| 9 | Skillet chain was changed. |
| 10 | Bearings were changed (6204, 2). |
| 11 | Ju 1367 skillet chain gears were changed (2). |
| 12 | Skillet chain apparatus gear shaft was changed. |
| 13 | Skillet gear looseness wwas corrected. |
| 14 | Bottom glue aplication roller was turned. |
| 15 | Bottom creasing roller was changed. |
| 16 | Fork was changed. |
| 18 | Delrin part supporting box carrriage was changed. |

| Inner box machine no 7 |  |
| --- | --- |
| 1 | Ju 1219 gear was changed (140, 1). |
| 2 | Ju 1218 gear was changed (141, 1). |
| 3 | Ju 1369 fiber gear gear was changed (Z 65-130, 1). |
| 4 | Bearing was changed (3306 T, 1). Fiber gear was changed. |
| 5 | Bearing was changed (6204, 3). |
| 6 | Ju 1349 chain gear was changed (2). |
| 7 | Ju 1367 gear was changed ( 2). |
| 8 | Ju 1356 skillet folders were changed (2). |
| 9 | Pins of the carriage were changed (90). |
| 10 | Carriage chain looseness was corrected. |
| 11 | Upper track was turned and knurled. |
| 12 | Fork was installed. |
| 13 | Flap dedector was installed. |
| 14 | Bottom glue applicator delrin was changed. |
| 15 | Bottom support guiding carriage was changed. |
| 16 | Pins of the carriage were changed (90 pieces). |

| Inner box machine no 8 |  |
| --- | --- |
| 1 | Fiber gear and bearing was changed (3306, 1). |
| 2 | Creasing shaft and bearing were changed (22207, 1). |
| 3 | Bearing was changed (6206, 2). |
| 4 | Ju 1218 gear was changed (141, 1). |
| 5 | Ju 1219 gear was changed (140, 1). |
| 6 | Ju 1369 gear was changed (Z65, 1). |
| 7 | Bearing was changed (6204, 2). |
| 8 | Bearing was changed (6203, 2). |
| 9 | Ju 1351 lower track was changed (1). |
| 10 | Ju 1350 gear and upper track were changed. |
| 11 | Ju 1367 gear was changed (124, 1). |
| 12 | Skillet chain wheel was changed (2). |
| 13 | Small box forming sides were changed (left-rigth). |
| 14 | Skillet chain main shaft was changed. New key was manufactured. |
| 15 | Loseness of big gear in front of carriage was corrected. |
| 16 | New chain was installed. |
| 17 | Lower diamond bit was changed. Head looseness was corrected. |
| 18 | Lower creasing adjustment apparatus was grinded. Loseness of creasing shaft was corrected. Loseness of upper track shaft was corrected. |
| 19 | Pin of gear driving creasing shaft was changed. |
| 20 | Flap dedector was installed. |
| 21 | The position of the machine was changed. New conical pin was made and installed. |
| 22 | Press bearings were greased. Covers were adjusted. |
| 23 | Brass skillet transfer parts of forming side were changed. |
| 24 | Skillet carriage chains were corrected. |
| 25 | Lower sides were grinded. |
| 26 | The fork of the carriage was changed. |
| 27 | Flap dedector was installed. |
| 28 | Cardboard preheater unit was installed. |
| 29 | The pins of the carriage were changed (90). |

| Inner box machine no 9 |  |
| --- | --- |
| 1 | Ju 1368 gear was changed (130, 1). |
| 2 | Bearings were changed (6204 2RS, 1). |
| 3 | Ju 1332 gear was changed (Z 21-131, 1). |
| 4 | Ju 1350 gear was changed (129, 1). |
| 5 | Ju 1351 F1 bottom track was changed (60, 1). |
| 6 | Ju 1369 fiber gear was changed (Z 651-138, 1). |
| 7 | Ju 1219 gear was changed (140, 1). |
| 8 | Ju 1218 gear was changed (141, 1). |
| 9 | Ju 1650 F1 chain path was changed. |
| 10 | Bottom carriage compressin bearins were changed (62203 2RS, 24). |
| 11 | Bearings were changed (6206 2RS, 3). |
| 12 | Upper creasing shaft bushings were changed. |
| 13 | Carton path was grinded. |
| 14 | The box pusher was grinded and re-installed. |
| 15 | Skillet support was grinded, adjusted and re-installed. |
| 16 | Chain support was grinded, adjusted and re-installed. |
| 17 | Fork was installed. |
| 18 | Flap detector was installed. |
| 19 | Cardboard preheater unit was installed. |

| Inner box machine no 10 |  |
| --- | --- |
| 1 | Ju 1219 geear was changed (140, 1). |
| 2 | Ju 1369 fiber gear was changed. |
| 3 | Ju 1368 gear was changed (130, 1). |
| 4 | Ju 1351 the bottom track was changed. |
| 5 | Small box forming sides were changed (left-rigth). |
| 6 | Carriage was changed (1 group 4 pieces). |
| 7 | The fork was changed. |
| 8 | Delrin part supporting box carrriage was changed. |
| 9 | Bottom diamond bit was changed. |
| 10 | Bottom glue aplication roller was changed. |
| 11 | Pins of the carriage were changed (90). |
| 12 | Chain crank-links were changed. (45 pieces) |
| 13 | Lancet knife was changed. |
| 14 | Flap dedector was installed. |
| 15 | Cardboard preheater unit was installed. |

| Inner box machine no 11 |  |
| --- | --- |
| 1 | Ju 1369 fiber gear was changed (Z 65-130, 1). |
| 2 | Ju 1218 fiber gear was changed (140, 1). |
| 3 | Ju 1218 fiber gear was changed (141, 1). |
| 4 | Bottom diamond bit was changed. |
| 5 | Lancet knife was changed. |
| 6 | Five monts ago maintenances of this machine had been completed. |

32.  Electrical Works

| 1 | Alarm photocells were installed to the Fatma-Sibel-Gülin KEB5A. |
| --- | --- |
| 2 | The reserve VAP machine was installed to the Sibel Continue line. |
| 3 | PP units were installed to the Gülin and Zeynep VAP machines. |
| 4 | Flap dedectors were installed to the 6th, 7th and 8th inner box machines. Electrical re-installation of the 7th, 8th and 9th inner box machines were done, because of the position change of these machines. |
| 5 | İç Because of the position change of the inner box machines, new electrical installation for lighting was done. |
| 6 | The bar mat security switch of the Gülin match machine was changed. |
| 7 | Paper pulling motor of the OPP unit of Fatma VAP machine was changed. |
| 8 | Impregnation liquid sprayer system was installed to the second line. |
| 9 | The breake alarm of the personnel was connected to the computer aided personnel entry control system. |
| 10 | Start warning ring of the Chambon 500 was repaired. |
| 11 | The VAP machine that was demontaging from the Sibel continues line was re-installed as reserve. |
| 12 | The motor of the chopping machine of the first line was changed. |
| 13 | The vibration motor of Ayşe KEB-5A was demontaged and the shaft of the motor was welded and resinstalled. |
| 14 | The saw motor of the first line was changed because of beeing damaged. |
| 15 | The damaged.aspiration motor of the transformer room was changed with its spare. |
| 16 | Cemile VAP machine OPP fhotocell were changed. |
| 17 | The fan motor of Cemile Fiv at the splint line was changed with its spare. |
| 18 | All the broken and damaged lighting switches in the working field were changed. |
| 19 | All the flourescent lamp in the working field were cleaned. |
| 20 | All the lighting lamps in the splint line were cleaned. |
| 21 | Maintenance and the prufication of the Varimatch microwave dryer were done. |
| 22 | Cemile Fiv motor was changed. |
| 23 | Splint leveling motor was installet to the inlet of no 2 dryer. |
| 24 | Electrical installation of Feride KU-1 was done. |
| 25 | Two-flourescent light was installed to the Sibel VAP machine. |
| 26 | 5 new flourescent lights was installed over the stripping machine in the splint line. |
| 27 | Heater lamps were installed to the 7th and 8th inner box machines. |
| 28 | All the splint line electrical cabinets were cleaned with compressed air. |
| 29 | All the elektrical cabinets in the work field were cleaned with compressed air. |
| 30 | All missing signal lamps were completed. |
| 31 | The deodorization ventilator of the kitchen was changed because of beeing damaged. |
| 32 | Dust filters of the electrical cabinets of inner box machine, outher box machine and buffer of Emine continue line has changed. |
| 33 | Dust filter of the frequency driver cabinet of the stripping machine in the splint line was changed. |
| 34 | General prufication of the generators was done. |

33. Other Works

| 1 | Upper sides of splint bins have been cleaned with pressure air. |
| --- | --- |
| 2 | At boiler room, inner surface of dustbins have been cleaned. |
| 3 | Ventilation ducts have been cleaned. |
| 4 | Condenser coils of big compressor have been cleaned. |
| 5 | New inner box transfer duct has been installed for Feride machine. |
| 6 | New splint transfer duct has been installed for Feride machine. |
| 7 | Upper plates of dryer 1 have been renewed with the isolated ones. |
| 8 | Paraffin tank and heater coils have been renewed with the bigger one. |