MAN ROLAND PERIODIC MAINTENANCE FORM

Operator: ........................Foramen: ...............................Maintenance Chief: ........................  Date:……........……

| Code | Maintenance Area | Job | Mon | Tue | Wed | Thu | Fri | Sat | ## Responsible |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 101 | Carriage chain | Lubricate | x | x | x | x | x |  | Operator |
| 102 | Red lubrication points | Lubricate | x | x | x | x | x |  | Operator |
| 103 | Red greasing points | Grease | x | x | x | x | x |  | Operator |
| 104 | Blue lubrication points | Lubricate | x | x | x | x | x |  | Operator |
| 105 | Blue greasing points | Grease |  |  |  |  | x |  | Operator |
| 106 | Compressor | Clean the air filter |  |  |  |  | x |  | Operator |
| 107 | Water reservoir | Clean |  |  |  |  | x |  | Operator |
| 108 | General cleaning | Clean |  |  |  |  | x |  | Operator |

#### Montly Maintenances

1)	Yellow lubrication points will be lubricated.

2)	Yellow greasing points will be greased

Every Three Months Maintenances

1)	Ink reservoir moving arm will be cleaned with gasoline.

Every  Six Months Maintenances

1)	Inner roller moving parts will be lubricated.

2)	Outer roller springs will be lubricated.

3)	Outer roller arms will be greased.

4)	Apparatus chain will be lubricated.

5)	Eccentric parts inside the apparatus will be greased.

6)	Ink receiver roller bearing will be greased.

Annual Maintenances

1)	Water rollers and bearings will be renewed.

2)	Ink rollers will be checked, if necessary will be recovered. The bearings will be renewed.

3)	Front and back escalator bushings will be renewed.

4)	Air filters will be renewed.

5)	Central lubrication system will be checked.

6)	Ink suction arm will be renewed.

7)	Gripper tips of carriage unit will be renewed for every three years.