# OILS FOR PURPOSES

| MOBİL | BP | EXXON | BELGİN | WHERE TO USE |
| --- | --- | --- | --- | --- |
| Mobil vacouline 1409 |  | Esso Nuto 100 |  | Chain, gear, cam, roller, bushing, guide |
| Mobil 80W–90 | ENERGEAR HYPO 80W/90 |  |  | Reducer, gearbox |
| Shell tellus C 10 | ENERGOL HP 10 |  |  | Pneumatic |
| Shell tellus 68 | ENERGOL HLP-HM 68 |  |  | Man Roland (winter) |
| Mobil special grease | ENERGREASE L21M |  |  | General bearings, rollers |
| Mobil gear 629 | ENERGOL GR-XP 150 |  |  | Chopping machine gearbox |
| Mobil gear 626 | ENERGOL GR-XP 68 |  |  | Piv variator |
| Mobil gear 632 | ENERGOL GR-XP 320 |  |  | Hand wheel gearbox |
| Mobil DTE 25 | ENERGOL HLP-HM 46 |  |  | Hydraulic units, Goebel |
| Mobil rarus 424 | ENERGOL RC-R 32 |  |  | Air compressor |
| Mobil DTE 24 BB | ENERGOL HLP-HM 32 |  |  | Blower oilı (waste treatment) |
|  | ENERGEAR 85 W/140 |  |  | Index gearbox |
|  |  |  | GRESON SD 000 | Liquid grease (for sealing problem reducers, ex: polishing drum reducer) |
|  |  |  | GRESON GP 72 | High quality grease (for important places that can not be removed frequently) |
| Mobil 8620 TS 1081 |  |  |  | Waste treatment |
| Mobil 1330 (SAE 30) |  |  |  | Transportation (cars, trucks) |
| BP super 20/50 |  |  |  | Transportation (cars, trucks) |
| Boron oil |  |  |  | Workshop machines |