SPLINT CHARGING ADJUSTMENT

- The stationary plastic cover under the movably wings at FOD unit is removed. A metallic sheet iron is to put between the straight splints (not to disturb all stacked splints).

- The supporting comb and the pressure combs were removed. The carrier comb bar is removed by releasing connection points and taken out.

- Splints on the grooved plate are cleaned. The grooves are cleaned.

- Disengaging lever is put on forward means in ‘charging position’. Make sure the roller is touching on the open cam. 30 mm studs are loosened by spanner.

- Rotate the machine by hand wheel in a manner that grooved plates and cleaning combs are in same position. The gap between them is checked by adjustment gauge. In case of adjustment need, it can be done by the screws (up or down) which are on the charging lever. Adjustment gauge should enter freely. Counter nuts are tightened.

- The machine is rotated a turn to bring the combs to the same position and checked again by the gauge.

- The machine is rotated a turn by hand wheel and stopped until the space between bar mat and grooved plate is to be 20 mm. In that position the gap of the stud is done. The nut is released and 0,1 mm gap is given by checking a feeler gauge. Then it is tightened by the counter nut. This adjustment is done for both sides. After rotating a turn, same checking are done for both side by using filler gauge.

- Carrier comb bar is handled from the it’s claws and put on lever and tightened the screws by hand.

- The machine is rotated a turn by hand wheel and stopped when the carriage comb is upward position. There should be only a loose fıt between carriage comb and grooved plates. It can be adjusted by using bottom screws and then be tightened counter. The screws of carrier comb bar are tightened tightly by spanners.

- The metallic sheet iron is taken away smoothly and splint stacking is done again.

- The supporting combs are placed again.

- The pressure combs are placed again

- The machine is rotated by hand wheel and stopped when the splints come under the pressure comb. The clearance between pressure comb and splint is checked visually in a manner that it will pass tangent. Its adjustment can be done by using the side screws.

- The machine is rotated by hand wheel and checked again.

- If everything is proper, the machine is run.