WEEKLY CLEANING PROGRAM

INNER BOX MACHINE (JUL2):

- Clean the machine with compressed air and waste cloth pieces (thread material only)

- Remove the glue pot; remove all the glue & any particles. After cleaning put it back.

KEB6:

- Clean the machine with compressed air

KU5 BOX CONVEYOR:

1.   Clean the machine with compressed air

OUTER BOX MACHINE (HUF/ABM):

- Clean the machine with compressed air and waste cloth pieces (thread material only).

- Remove the glue pot; remove all the glue & any particles. After cleaning put it back.

- (once in a month) The plates under carrier chain should be removed. The springs, shafts and sleeves should be cleaned. After cleaning fix them again.

- Clean pressure rails and guides, after cleaning put them again.

- Lubricate springs, shafts, sleeves, rails and guides.

OUTER BOX RANGING DEVİCE (KED4):

- Clean the machine with compressed air and waste cloth pieces (thread material only).

ROG2:

1.   Clean the machine covers with compressed air and dry clothes pieces.

2.   Apply compressed air to cams and shafts then clean dirt and oil by dry clothes pieces.

3.   Clean up the top part of box carrier steel band by dry clothes pieces. Then follow the same practice for the bottom part of the carrier.

4.   Open top covers of steel band and clean.  Then follow the same practice for the bottom part of the carrier with dry clothes pieces (one complete cycle).

5.   Lubricate box carrier steel band.

KL2:

1.   Clean the machine with compressed air starting from up to down then use dry clothes pieces (except in bar mat chains – this is very important).

PARAFFIN UNIT (MINIMUM ONCE A MONTH):

1.   Melt all the paraffin & remove it by opening bottom valve.

2.   Remove connections and take the paraffin tank out.

3.   Clean inside the tank with compressed air then use dry clothes pieces.

4.   Put the tank into the place again.

5.   Return the paraffin back into the tank

DIPPING UNIT:

1.   Remove connections and take the dipping table out.

2.   Clean very well the sides of table and conveyor belt by using steam to make sure

all dried chemicals which can not be cleaned during shift hours are removed.

3.   Check whether the conveyor belt bearings have worn out.

4.   Put the table into the place again.

INSPECTION TABLE:

1.   Clean the machine with compressed air and dry clothes pieces.

BOX CONVEYOR (KU1):

1.   Clean the machine with compressed air and dry clothes pieces.

WRAPPING MACHINE (VAT):

1.   Clean the machine with compressed air and dry clothes pieces.

2.   Clean dirt and oil on cams and other parts inside the unit.

3.   Remove the glue pot, after cleaning put it again.

SHRINK WRAPPING MACHINE (VYB 800):

- Clean the machine with compressed air and dry clothes pieces.

ALL THE UNITS MUST BE FINISHED & LEFT IN A FULL RUNNING CONDITION SUCH THAT ON MONDAY MORNING THERE IS NO DOWN TIME

REQUIRED PERSONNEL AREAWISE